|  | The Gambia Meteorological Services Department of Water Resources 7 Marina Parade |
| --- | --- |

## Monthly Climate Bulletin

N° 6 Jan-Feb-Mar 2015

1.0   Station history of The Gambia Meteorological Network & rainfall stations

The Gambia Meteorological service used to operate fifteen (15) Network stations covering all the regions in the country, but these was reduced to ten (10) in 2008. As of now, the department operates 10 synoptic weather stations and 25 rainfall statons as shown in table 1.

  - Network of Meteorological and Rainfall stations

Figure 1 shows Network of Meteorological stations in black symbols, 24 hour stations in blue symbols and red symbols for stations closed in 2008. The green symbols are the proposed new stations in 2014.

- Day light stations

- 24 hour stations

- Stations closed

- Proposed new Stations in 2014

Figure 1 : Network of Meteorological station

Table 1: Network of  Meteorological and Rainfall stations

| No. | STATION- CODE | STATION-NAME | BEGIN-DATE | END-DATE | STATION_QUALIFIER | LAT- | LAT- | LON- | LON- | ELEVATION | REMARKS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DEGR | MINU | DEGR | MINU | (m) |  |
| 1 | 15000200 | Pirang mfc. | 01/01/1971 | Todate | Rainfall station | 13 | 16 | 16 | 32 | 7 |  |
| 2 | 15000500 | Serrekunda | 01/01/1977 | Todate | Rainfall station | 13 | 26 | 16 | 39 | 4 |  |
| 3 | 15000600 | Kaur met. | 01/01/1984 | Todate | Met.station | 13 | 42 | 15 | 20 | 11 |  |
| 4 | 15000800 | Naude mfc. | 01/01/1971 | Todate | Rainfall station | 13 | 28 | 14 | 27 |  |  |
| 5 | 15001100 | N'geyen sanjal mfc | 01/01/1970 | Todate | Rainfall station | 13 | 36 | 15 | 26 |  |  |
| 6 | 15001200 | Basse met. | 01/01/1949 | Todate | Met. station | 13 | 19 | 14 | 13 | 4 |  |
| 7 | 15001400 | Kwinella mfc. | 01/01/1971 | Todate | Rainfall station | 13 | 24 | 15 | 48 |  |  |
| 8 | 15001500 | Gibanack mfc. | 01/01/1971 | Todate | Rainfall station | 13 | 13 | 16 | 11 | 9 |  |
| 9 | 15001600 | Sare n'gai mfc. | 01/01/1974 | Todate | Rainfall station | 13 | 29 | 14 | 50 |  |  |
| 10 | 15001700 | Jassong mfc | 01/01/1971 | 31/12/1999 | Rainfall station | 13 | 24 | 15 | 18 |  | Closed |
| 11 | 15002400 | Bansang | 01/01/1951 | Todate | Rainfall station | 13 | 26 | 14 | 40 |  |  |
| 12 | 15002500 | Jakunda mfc. | 01/01/1974 | Todate | Rainfall station | 13 | 29 | 14 | 11 |  |  |
| 13 | 15002900 | Dankunku mfc | 01/01/1971 | Todate | Rainfall station | 13 | 34 | 15 | 18 |  |  |
| 14 | 15003000 | Banjul Halfdie/Marina | 01/05/1943 | Todate | Met. station | 13 | 27 | 16 | 34 | 2 |  |
| 15 | 15003100 | Sare sofi mfc. | 01/01/1971 | Todate | Rainfall station | 13 | 25 | 14 | 31 |  | Met. closed |
| 16 | 15003300 | Kerewan met. | 01/07/1974 | Todate | Met. station | 13 | 30 | 16 | 6 | 15 |  |
| 17 | 15003500 | Keneba mrc | 01/05/1974 | Todate | Rainfall station | 13 | 20 | 16 | 1 |  |  |
| 18 | 15003700 | Kanjibat mfc. | 01/01/1971 | Todate | Rainfall station | 13 | 13 | 15 | 58 | 8 |  |
| 19 | 15003900 | Georgetown met. | 01/01/1949 | Todate | Met. station | 13 | 32 | 14 | 46 | 1 |  |
| 20 | 15004000 | Nja'bakunda mfc | 01/01/1971 | Todate | Rainfall station | 13 | 33 | 15 | 55 | 8 |  |
| 21 | 15004100 | Bakendik mfc | 01/01/1971 | Todate | Rainfall station | 13 | 27 | 16 | 27 | 9 |  |
| 22 | 15004300 | Jambanjelly mfc/met. | 01/01/1971 | Todate | Rainfall station | 13 | 17 | 16 | 44 | 7 | Met . closed |
| 23 | 15004800 | Mamudfana mfc. | 01/01/1971 | Todate | Rainfall station | 13 | 36 | 15 | 7 |  |  |
| 24 | 15004900 | Jenoi met. | 01/09/1974 | Todate | Met.station | 13 | 28 | 15 | 34 | 20 |  |
| 25 | 15005000 | Kuntair mfc | 01/05/1976 | Todate | Rainfall station | 13 | 32 | 16 | 13 |  |  |
| 26 | 15005100 | Somita mfc. | 01/01/1972 | Todate | Rainfall station | 13 | 12 | 16 | 18 | 7 |  |
| 27 | 15005300 | Jali mfc. | 01/01/1974 | Todate | Rainfall station | 13 | 21 | 15 | 58 |  |  |
| 28 | 15005400 | Yundum Inter Airport | 01/08/1945 | Todate | Met. station | 13 | 21 | 16 | 38 | 33 |  |
| 29 | 15005500 | Yallal mfc./met | 01/01/1971 | Todate | Rainfall station | 13 | 33 | 15 | 43 |  | Met. closed |
| 30 | 15005600 | Girobakunda | 01/01/1973 | Todate | Rainfall station | 13 | 18 | 14 | 11 |  |  |
| 31 | 15005800 | Njau mfc. | 01/01/1971 | 31/12/1999 | Rainfall station | 13 | 45 | 15 | 12 |  |  |
| 32 | 15005900 | Sibanor met. | 01/01/1986 | Todate | Met. station | 13 | 13 | 16 | 12 | 21 |  |
| 33 | 15006000 | Mankamang mfc. | 01/01/1974 | Todate | Rainfall station | 13 | 20 | 14 | 26 |  |  |
| 34 | 15006200 | Sapu met. | 01/01/1956 | Todate | Met. station | 13 | 33 | 14 | 54 | 3 |  |
| 35 | 15006500 | Farafenni mrc | 01/01/1984 | 31/12/1999 | Rainfall station | 13 | 35 | 15 | 38 |  | Closed |
| 36 | 15006600 | Fatoto disp./met. | 01/01/1976 | Todate | Met. station | 13 | 24 | 13 | 53 | 17.5 |  |
| 37 | 15006800 | Kiyang west mfc/met | 01/01/1978 | Todate | Rainfall station | 13 | 21 | 16 | 9 |  | Met. closed |
| 38 | 15006900 | Kuntaur met. | 01/01/1986 | Todate | Met. station | 13 | 40 | 14 | 53 | 7.2 | Met. closed |
| 39 | 15007100 | Sutukoba mfc | 31/05/1971 | Todate | Rainfall station | 13 | 30 | 14 | 1 |  |  |

  - Part 1: Rainfall in millimetres (mm) for January-February-March 2015

| Table 2 : Rainfall in millimetres (mm) for Synoptic & Rainfall stations |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Station | R_tot | Normal_1981-2010 | R_Max | R_Max_date | R_Min | R_Min_date |
| Banjul |  | 18.6 |  |  |  |  |
| Yundum | 0.5 | 22.1 | 0.4 | 18th March | 0.1 | 19th March |
| Kerewan |  | 23.4 |  |  |  |  |
| Sibanor |  | 24.4 |  |  |  |  |
| Kaur |  | 23.6 |  |  |  |  |
| Jenoi |  | 26.9 |  |  |  |  |
| Sapu |  | 35.5 |  |  |  |  |
| Janjanbureh |  | 31.6 |  |  |  |  |
| Basse | Tr | 39.2 | Tr | 20th March | Tr | 20 th March |
| Fatoto | 1.6 | 37.6 | 1.6 | 18th March | Tr | 20 th March |
| Bakendik |  | 16.2 |  |  |  |  |
| Kuntair |  | 33.4 |  |  |  |  |
| Ngain-Sanjal |  | 24.6 |  |  |  |  |
| Kuntaur |  | 33.4 |  |  |  |  |
| Jambanjelly |  | 17.5 |  |  |  |  |
| Pirang |  | 19.6 |  |  |  |  |
| Karantaba |  | 24.7 |  |  |  |  |
| Kwinella |  | 29.1 |  |  |  |  |
| Yallal |  | 23.7 |  |  |  |  |
| N’jau |  | 25.2 |  |  |  |  |
| Dankunku |  | 23.9 |  |  |  |  |
| Gibanak |  | 21.7 |  |  |  |  |
| Jahkunda |  | 30.8 |  |  |  |  |
| Bansang |  | 36.0 |  |  |  |  |
| Saresofi |  | 37.4 |  |  |  |  |
| Mankamang |  | 30.8 |  |  |  |  |
| Girobakunda |  | 33.6 |  |  |  |  |
| Jali |  | 15.3 |  |  |  |  |
| Kanjibat |  | 35.9 |  |  |  |  |
| Serrekunda |  | 16.4 |  |  |  |  |
| Sutukoba |  | 27.8 |  |  |  |  |
| Naude |  | 32.5 |  |  |  |  |

1.2  Part II : Temperature in degrees Celsius (0C) for January-February-March 2015

| Station _codes |  |  | MeanT-2015 | Normal 1981-2010 | MeanTmax- 2015 | Normal Tmax | MeanTmin-2015 | Normal Tmin | AbTmax-2015 | Normal AbTmax | AbTmin-2015 | Normal AbTmin |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WMO | National | Name |  |  |  |  |  |  |  |  |  |  |
| 61711 | 15003000 | Banjul Halfdie/Marina | 25.0 | 28.0 | 33.0 | 32.4 | 17.0 | 22.0 | * | 32.8 | 21.0 | 18.6 |
| 61731 | 15001200 | Basse | 27.5 | 27.1 | 37.1 | 37.4 | 17.8 | 24.9 | 39.9 | 39.6 | 22.4 | 15.0 |
| 61733 | 15006600 | Fatoto disp./met. | 26.3 | 27.2 | 37.1 | 37.6 | 15.5 | 24.3 | 40.5 | 40.0 | 19.7 | 15.4 |
| 61721 | 15003900 | Georgetown | 23.7 | 27.1 | 37.2 | 37.1 | 10.2 | 23.7 | 37.2 | 39.4 | 19.6 | 15.3 |
| 61707 | 15004900 | Jenoi | 26.8 | 26.8 | 35.7 | 36.8 | 15.6 | 22.4 | 38.7 | 38.8 | 20.3 | 14.4 |
| 61717 | 15000600 | Kaur | 27.5 | 27.6 | 36.2 | 36.8 | 18.8 | 23.0 | 39.4 | 38.9 | 23.7 | 17.2 |
| 61712 | 15003300 | Kerewan | 25.4 | 27.3 | 35.5 | 36.8 | 15.1 | 21.6 | 38.9 | 38.6 | 18.5 | 14.3 |
| 61722 | 15006200 | Sapu | 24.1 | 26.6 | 35.5 | 36.6 | 17.6 | 23.4 | 38.9 | 38.9 | 21.4 | 14.7 |
| 61705 | 15005900 | Sibanor | 25.0 | 26.6 | 35.7 | 36.0 | 14.3 | 21.7 | 39.3 | 38.1 | 17.6 | 16.1 |
| 61701 | 15005400 | Yundum Airport | 25.4 | 26.7 | 33.1 | 33.8 | 17.7 | 21.5 | 38.1 | 34.7 | 20.7 | 16.9 |

  - Part III: Relative Humidity in percentage (%) and Wind in metres per second (m/s) for

January-February-March 2015

| Station Codes |  |  | Relative Humidity (%) |  |  |  | Windspeed (m/s)_2015 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WMO | National | Name | MeanRH_2015 | 1981-2010 | MaxRH_2015 | MinRH_2015 | Mean | Highest |
| 61711 | 15003000 | Banjul Halfdie/Marina | 41 | 63 | 85 | 50 | 4.5 | 8 |
| 61731 | 15001200 | Basse | 31 | 64 | 73 | 25 | 2.9 | 15 |
| 61733 | 15006600 | Fatoto disp./met. | 24 | 57 | 67 | 33 | 5.0 | 18 |
| 61721 | 15003900 | Georgetown | 38 | 60 | 85 | 43 | 3.9 | 13 |
| 61707 | 15004900 | Jenoi | 29 | 57 | 80 | 34 | 4.7 | 14 |
| 61717 | 15000600 | Kaur | 24 | 52 | 83 | 35 | 6.2 | 19 |
| 61712 | 15003300 | Kerewan | 32 | 59 | 81 | 31 | 8.8 | 15 |
| 61722 | 15006200 | Sapu | 26 | 58 | 69 | 21 | 8.6 | 12 |
| 61705 | 15005900 | Sibanor | 40 | 63 | 90 | 40 | 4.9 | 17 |
| 61701 | 15005400 | Yundum Airport | 47 | 70 | 92 | 43 | 7.9 | 21 |

1.5     Part IV: Sunshine (hours) and Evaporation (mm) for January-February-March 2015

| Station_codes |  |  | Sunshine 2015 |  |  |  | Evaporation Class A (mm) 2015 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WMO | National | Name | Total | Normal_1981-2010 | Max | Min | Total | Mean |
| 61711 | 15003000 | Banjul Halfdie/Marina | * | * | * | * | * | * |
| 61731 | 15001200 | Basse | 209.9 | 192.2 | 10.5 | 4.0 | 194.4 | 6.5 |
| 61733 | 16006600 | Fatoto disp/met. | 217.2 | 86.7 | 10.5 | 4.3 | 275.6 | 9.2 |
| 61721 | 15003900 | Georgetown | 228.5 | 147.1 | 10.5 | 2.4 | 224.7 | 7.2 |
| 61707 | 15004900 | Jenoi | 239.9 | 181.5 | 10.7 | 3.4 | 204.2 | 6.8 |
| 61717 | 15000600 | Kaur | 245.1 | 99.7 | 10.7 | 2.3 | 299.0 | 10.0 |
| 61712 | 15003300 | Kerewan | 229.8 | 201.8 | 10.4 | 2.7 | * | * |
| 61722 | 16006200 | Sapu | 234.8 | 125.9 | 10.5 | 4.3 | 234.3 | 8.0 |
| 61705 | 15005900 | Sibanor | 222.8 | 116.5 | 10.4 | 2.1 | 131.6 | 4.7 |
| 61701 | 15005400 | Yundum Airport | 268.5 | 218.5 | 10.7 | 3.4 | 189.1 | 6.3 |

2.0     Part V:   Climate Report explanations

PREFACE

Selected climatological data of general importance from weather stations in the Gambia has been compiled in this report. The values have been checked. Blank spaces indicate either missing or delayed records. If data are missing one or several days for one station within the selected period, this station are marked with an asterix (*). Observed values (OBS) and the normals (NORM) of these data values from the long period averages are given for the following elements :

- Average air temperature

- Rainfall

- Relative Humidity

- Sunshine

- Interpolated map for rainfall is included.

EXPLANATION OF TABLES

- Rainfall (mm) amount is measured every six hours in a standard rain gauge 80cm above ground level. The Total rainfall amount for any given day is the cumulative or accumulated six hourly rainfall amounts. A Rainy day is defined as a day with a total rainfall amount equal or more than 1mm.

- Air Temperature (°C) observations are made from thermometers exposed in Stevenson Screens, 1.25 metres above the ground. The Maximum and Minimum Temperature (°C) observed/ obtained, indicates the highest and lowest temperatures during the specified period.

- Relative Humidity is derived from the readings of the dry and wet bulb thermometers and expressed in Percentage (%).

- Sunshine Duration is measured using a Campbell-Stokes Tropical Sunshine Recorder and is recorded in hours.

- Actual Evaporation is determined from a Class A Pan and measured in millimetres (mm).

- Wind speed per day is read from a Cup Counter Anemometer mounted 10 metres above the ground and read in knots. It can also be converted to metres per second (m/s) or kilometer per hour (km/h).

- N.B : January, February, March, April, November and December are dry season months in The Gambia and any rainfall that occurs during this period are considered as unseasonal rainfall.

Abbreviations :

- MeanT : Mean Temperature

- Mean Tmax : Mean Maximum Temperature

- Mean Tmin : Mean Minimum Temperature

- AbTmax : Absolute Maximum Temperature

- AbTmin : Absolute Minimum Temperature

Published by: Climate Unit

Department of Water Resources

7 Marina Parade

Banjul, The Gambia

Telephone:  00 (220) 422-7631/4222-872/422-4122

Fax: 	 00 (220) 422-5009

http://www.mofwrnam.gov.gm

E-mail: info@mofwrnam.gov.gm; infoclimate@gmail.com